O-Scope I Demonstration Program

(c) 1994 Allison Technology Corporation

8343 Carvel  Houston  TX  USA  77036

PH: (713) 777 0401  FAX/BBS: (713) 777 4746

DEMO1P.EXE is a demonstration program for O-Scope I and O-Scope

Ip operations.  THIS PROGRAM IS PROVIDED FREE OF CHARGE WITH A

LIMITED LICENSE. YOU MAY FREELY COPY AND DISTRIBUTE THIS PROGRAM

IN A COMPLETE AND UNMODIFIED FORM.  YOU MAY PRINT THIS TEXT FILE

AND USE DEMO1P.EXE FOR A PERIOD OF 15 DAYS FOR THE PURPOSE OF

EVALUATING OSCOPE I PRODUCTS.

CONTACT ALLISON TECHNOLOGY CORPORATION FOR INFORMATION ON O-SCOPE

I, Ip, AND ADDITIONAL PRODUCTS FOR TRAINING AND EDUCATION.

Introduction

O-Scope I is a low cost module that plugs into a PC's printer

port and provides digital storage oscilloscope capabilities.

DEMO1P.EXE is a demonstration and training program for the

O-Scope I and Ip which does not require the hardware module.  All

that is needed to run DEMO1P.EXE is a PC AT compatible computer

with a VGA, EGA, or Monochrome graphics display.

DEMO1P.EXE can generate a variety of sine waves, square waves,

and DC signals to view.  In addition, O-Scope Ip log files can

also be viewed.  Its operation for the internally generated wave

forms is very close to the real time performance experienced when

capturing data using the O-Scope hardware pod.

Two demonstration modes are provided.  These are O-Scope I and

O-Scope Ip.  The first, O-Scope I, demonstrates the capabilities

of the standard O-Scope software package.  The second, O-Scope

Ip, demonstrates the capabilities of the enhanced software

package.

The actual program is based on the O-Scope Ip version and there

are a few differences between the DEMO1P.EXE O-Scope I demonstra-

tion and the actual O-Scope I software.  The most notable of

these differences is the log file.  Both demonstrations actually

use the O-Scope Ip log file version which provides substantially

more header information than does the O-Scope I log file.

Starting DEMO1P.EXE

DEMO1P.EXE can be run from either a floppy disk or a hard disk

drive.  To install it on the hard drive, create a subdirectory

and copy the files to that subdirectory.

=================================================================

Example 1 - Set up the demonstration on hard drive c: from floppy

drive b:

C:\>md \oscp<Enter>

C:\>cd oscp<Enter>

C:\OSCP>copy b:*.*<Enter>

Example 2 - Run the demo program from subdirectory C:\OSCP

C:\>cd oscp<Enter>

C:\OSCP>demo1p<Enter>

=================================================================

Once DEMO1P.EXE has been started, it will print a screen full of

information about its use and wait for you to select which demo

to run.  If you have your printer attached to your PC you may

print this screen out with the <Prt Sc> key.  See Appendix A for

the procedure on printing out graphics screens.  To select the

O-Scope I demo,enter 0.  To select the O-Scope Ip demo, enter 1.

Pressing any other key will begin the O-Scope Ip demo.  Except

for the demonstration waveform control keys 5 through 9 and 0,

DEMO1P.EXE works and looks like the O-Scope programs.

There are 3 waveforms types available in the demo.  These are

sine wave, square wave, and DC.  The 0 key selects DC, 9 key

selects the square wave, and the 8 key selects a sine wave.

The sine wave and square wave have a frequency which is selected

by the 5 key.  Each time the 5 key is pressed the next highest

frequency will be selected until the highest has been reached.

Then the lowest frequency is selected.  There are 10 frequencies

which range from 0.179 Hz up to 38,000 Hz.

A peak magnitude is also associated with the sine and square

waves.  The sine waves vary between the (-) peak voltage and the

(+) peak voltage.  Square waves vary between 0 Volts and the peak

voltage.  A DC offset voltage is also added to these waveforms

although its value can be set to 0.  The peak voltages (or

magnitudes) range from 0.024 Volts to 27.5 Volts.  Again, there

are 10 voltages which are changed by pressing the 6 key.  Each

time the 6 key is pressed, the voltage increases to the next

higher value.  When the highest value is reached, it starts over

at the lowest.

A DC voltage (or DC offset voltage) is controlled with the 7 key.

There are 10 different voltages starting with 0.0 Volts and

increasing to 28.0 Volts.  Each time the 7 key is pressed the DC

voltage is increased to the next higher value until the highest

is reached.  The voltage is the set to 0.0 again.

----------------------------------------------------------------

TABLE 1 - Wave forms (* starting values)

Frequencies(5 key)      Peak Voltages (6 key)   DC Voltages (7 key)

Hz                      Volts                   Volts

0.179                   0.024                   0.0

5.10 *                  0.14                    0.05

91                      0.55                    0.15

170                     1.2                     0.50 *

240                     2.2                     1.0

680                     3.8                     3.8

1,210                   5.1                     5.2

9,638                   6.2 *                   12.0

19,205                  12.3                    18.0

38,420                  27.5                    28.0

----------------------------------------------------------------

O-Scope I Operation (Both I and Ip)

There are 2 windows for O-Scope I and 4 windows for the O-Scope

Ip.  The top window is referred to as the text window.  The large

bottom window is called the graphics window.  On O-Scope Ip the

smaller bottom windows to the left and right of the graphics

window are the sweep window and the cursor window.  Most settings

are done using the cursor keys.  Up and Down Arrow keys move the

cursor marker, "*>", up and down to the various functions.  The

Left and Right Arrow keys then can be used to change the settings

for the functions.  Some functions use a keystroke such as A-key

to select AC coupling and D-key for DC coupling.

In addition to the Left and Right Arrow keys, the <Pg Up> and

<Pg Dn> keys will change values faster.  Each key stroke for

<Pg Up> and <Pg Dn> is the equivalent of 10 key strokes for the

Left and Right Arrow keys.

----------------------------------------------------------------

Table 2 - Key Functions

Key     Description

A       Select AC coupling

B       Begin Spectrum Averaging

C       Toggle connecting lines between sample points

D       Select DC coupling

E       Erase graphics window

F       Toggle between Spectrum and signal sweeps

L       Toggle data logging

M       Bring up main menu (O-Scope Ip only)

P       Expand the sweep using DSP mode X2, X4, ... X16

R       Roll mode (O-Scope Ip only)

S       Single sweep (O-Scope Ip only)

T       Move cursor to top of text window

V       Toggle View log mode (O-Scope Ip only)

X       Expand sweep by spreading out sample points X1, X2, X5

<Space> Toggle between pause/review mode and sweep mode

<Esc>   Exit Program

----------------------------------------------------------------

Basic Settings

There are only 3 basic settings for the O-Scope.  These are the

vertical and horizontal sweep and the AC/DC coupling.  The

vertical sensitivity sets the input voltage required to move the

trace up or down by 1 division.  The horizontal sweep sets the

time that the sweep takes to go through 1 division.  The proper

settings for these 3 items should allow you to see the wave form.

Beyond these very basic settings are a few more provided for

versatility.  Trigger, hold time, and vertical position help make

the O-Scope more useful.  Beyond these are the O-Scope Ip

settings.

Vertical Sensitivity

Vertical sensitivity sets the volts per division.  It indicates

the volts necessary to move the trace up or down by 1 division.

O-Scope ranges go from 10 Volts/Division down to 50

millivolts/Division.  Positive voltages raise the sweep and

negative voltages lower the sweep.  With a standard x1 scope

probe, the maximum voltage measurable is +/- 30 volts peak.

O-Scope will handle standard attenuating probes such as x10 and

x100.  These will allow the maximum readings to be +/- 300 volts

peak and +/- 3000 volts peak, respectively.  These probes must be

adjusted for proper operation.  The normal adjustment procedure

is to look at a 1 kHz, 1 volt square wave and adjust the wave

form until it is square.  NOTE THAT VOLTAGES IN EXCESS OF 50-60

VOLTS ARE CONSIDERED DANGEROUS AND CAN BE DEADLY.  PROPER CARE

SHOULD BE TAKEN.

Sweep Rates

Sweep rate determines the time span of each horizontal division.

One adjusts this value to display the wave form as desired.

O-Scope offers sweep rates ranging from 100 seconds per division

down to 500 microseconds per division at the standard x1

expansion.  In the normal x1 expansion mode this results in 50

samples per each division.

The X key cycles between x1, x2, and x5 expansion modes.  This

spreads the samples apart which can help you see some wave forms

better.  The x2 mode has 25 samples per division and the x5 has

10 samples per division.  This also has the effect of increasing

the sweep rate per division and the value changes with the expan-

sion mode.

O-Scopes offer a second expansion mode called the DSP, digital

signal processing, mode cycled by the P key.  This offers x2, x4,

x8, and x16 expansions.  Rather than just spreading out the

sample points like the X key, the DSP mode creates the missing

samples by calculations based on the measured waveform.  Although

this wave form will be missing any components whose frequency is

above 1/2 of the sampling rate, it can provide significant im-

provements in viewing the higher range of O-Scope frequencies.

The trade off is a slowing down of the sweep cycle rate due to

the extensive calculations involved.  Use for 20ms/div and faster.

Part of the nature of a digital storage oscilloscope is the

possibility of aliasing.  Since samples are captured at a specif-

ic rate and the input is ignored between these samples, a wave

form at a frequency over half of the sample rate can appear as a

signal of much lower frequency.  This is called aliasing and it

obviously can provide false signal information.

On the O-Scope units, changing sweep rate changes the sampling

rate.  When viewing an aliased signal, it will change dramatical-

ly with different sweep rates (sample rates) where a normal

signal will simply spread out or contract as the sweep rate

changes.  It is good practice when viewing a signal of unknown

frequency to first increase the sweep rate to faster sweep times

to make sure that you are not viewing an aliased version of the

wave form.

AC/DC Coupling

AC/DC coupling selects whether or not you want to look at the DC

component of the voltage.  Selecting the AC mode places a capaci-

tor between the input signal and the electronics.  This allows

the AC signal portion to be viewed while subtracting out the DC

part.  Although AC signals below 10 Hz may be affected by AC

coupling, anything above that should not be affected.  AC cou-

pling is used when it is desired to see a small AC signal riding

on a DC value that would force you to reduce the vertical sen-

sitivity to see.  Looking at 0.25 Volts of ripple on a 12 Volt DC

level would be a typical example.  The DC mode would require at

least the 5 Volts/division setting and the 0.25 Volts of ripple

would amount to 1/20 of a division.  The AC mode would allow

readings down to 100 or even 50 millivolts/division where the

ripple size would be several divisions.

Vertical Position

Vertical position allows you to move the wave form above or below

the 0 line in the center of the graphics screen.  This is done so

that the wave form can be positioned by the user to a suitable

vertical position.  This setting has no effect on calculations or

the O-Scope's voltage range.

Hold Time

Hold time allows you to delay between sweeps.  When measuring

voltages in some systems, it is desirable to wait before captur-

ing a new sweep.  This time begins when the data is displayed.

It is shown in milliseconds (1/1000ths of a second).  It can be

set from 0 to 9,990 (9.9 seconds).  There is always some minimum

delay time, usually over 10 milliseconds.

Trigger

The trigger event begins a sweep.  It has a mode and a level.  If

the trigger is off, then the next sweep begins as soon as the

hold time runs out.  Trigger Slope + starts the trace when the

trigger signal rises past the trigger level.  Trigger Slope -

starts the trace when the trigger signal falls below the trigger

level.  The trigger level is adjusted and shown as % of full

scale.  Also immediately to the left of the graphics window (in

the sweep window) is a tick mark which indicates the trigger

point on the wave form.  This tick mark also moves with vertical

position so that it always shows the actual position on the wave

form.  NOTE, WHEN THE TRIGGER IS ACTIVE THE TRACE DOES NOT OCCUR

UNTIL THE WAVE FORM CROSSES THE LEVEL IN THE SELECTED SLOPE

DIRECTION.  TRACES WILL STILL BE STARTED WHEN ANY KEYS ARE

PRESSED.

O-Scope Ip Settings

O-Scope Ip offers a number of features not found in the standard

O-Scope I product.  Two additional windows have been provided to

contain these.  The first window, located to the right of the

graphics window, contains cursor settings.  The second, located

to the left of the graphics window contains various features

relating to sweep and calculations.

Cursors

A cursor is a straight line which goes from one side of the

graphics window to the other.  There are 2 voltage cursors and 2

time cursors available on the sweep display.  Each has a

corresponding read out value in the cursor window.  Cursor 1

consists of a solid line.  Cursor 2 is dashed.

A cursor is selected using the Up Arrow and Down Arrow keys.

Rather than a "*>" indicator to the left, the selected item is

highlighted in reverse video.  The cursor may be moved using the

Left Arrow and Right Arrow keys.  Also, the <Pg Up> and <Pg Dn>

keys will move the cursor much more rapidly.  <Pg Up> is the

rapid equivalent of Left Arrow and <Pg Dn> is the equivalent of

the Right Arrow.

The primary purpose of a cursor is to make a precise reading for

some part of a wave form.  When both cursors for time or voltage

are displayed on the graphics window, a difference called delta

is calculated.  Delta V is the difference between Volts cursors,

V1 - V2.  It can be positive or negative.  Delta T is the time

difference between the time cursors and will always be positive.

If all 4 cursors are on screen,  the DV/DT ratio between voltage

and time will be calculated.

A voltage cursor is a horizontal line which can be brought down

on to the display by highlighting the desired cursor window item

and pressing the Left Arrow key (the <Pg Up> key will move the

cursor in the same manor as 10 Left Arrow key strokes).  It may

be raised by pressing the Right Arrow key (or the <Pg Dn>).  The

cursor window reading will show the voltage value at the cursor

line.  Readings are corrected for the Volts/Div. setting and the

vertical position.

Time cursors are vertical lines which may be moved on screen from

the left side of the window using the Right Arrow (or <Pg Dn>.

Time cursor 1 is a solid line.  Time cursor 2 is dashed.

In the Spectrum mode, there are 2 frequency cursors and 2 power

cursors.  Delta calculations are not provided for these.  Power

cursors operate like voltage cursors and frequency cursors

operate like time cursors in the normal sweep.

Sweep Window

There are several different control items located in the Sweep

window.  First are the sweep size and location.  The next item is

a delayed sweep feature.  After that are two calculation settings

associated with probe attenuation and voltage offset.  The sweep

window also contains a status indicator showing the operational

state of the O-Scope.

#divisions

One of the most important features unique to O-Scope Ip is the

ability to extend the size of a sweep.  While the display screen

contains only 10 divisions, the sweep length can be extended to

hundreds of divisions.  The #divisions can be changed from 10

divisions up to several hundred.

While this increases the time it takes to perform a sweep, it

allows you to capture multiple events at much greater time

resolution than possible with only the 10 divisions available on

screen.

Running at 500 microseconds per division with only 10 divisions

means that you can only see what happens during 5 milliseconds.

Setting the sweep length to 200 allows you to view events at 500

microseconds per division for 0.10 seconds (1/10 of a second).

Location

Location is how you move from one part of the sweep to another

when the number of divisions is increased beyond 10.  Location

indicates where the left most division is in the sweep.  Location

0 means that the left most division is at the beginning of the

sweep.  Location must always be less than the number of divisions

- 10.

Location can be used in continuous sweeping mode to generate a

precision time delay from the trigger event (see below for delay

time).  To view an entire sweep you must enter the pause/review

mode using the <space bar> key.  Once in pause/review, you may

then move back and forth through the sweep by changing the

location.

Delay Time

Delay time provides 2 functions.  First, it is possible to add a

delay time between the trigger event and the beginning of data

capture.  Second, the delay time value indicates both this

trigger delay plus the delay time from the beginning of the sweep

until the current location.  This time is given in seconds.  A

value of 0 only occurs when there is no delayed trigger and when

the location is at 0.

Highlighting delay time and changing it will result in changing

the delay between trigger and the beginning of the sweep.  If the

trigger is off, this merely results in an increase in the time it

takes to do a sweep.  Delay time cannot be less than 0.  Delayed

sweep occurs in multiples of the sample time rate so it will

change with the sweep rate.

Probe X

Probe X provides a means of correcting the voltage calculations

for an attenuating probe such as an x10 or x100.  A voltage

reading of 1.5 volt measured when using an x10 probe means that

the actual voltage was 15 volts.  Setting probe x to 10 will

cause the calculations to show the real voltage of 15 volts.

Note that this does not affect the volts per division value or

the voltage cursor values.  NOTE in this demo program, probe x

does not work.

V Offset

In some readings it may be desirable to offset the voltage used

for calculations.  Besides using it to correct for a minor

adjustment for the O-Scope scale, it can be used to correct for

signal readings.  In the case of a 0.25 volt ripple on a 12 volt

DC power supply, the display may be AC coupled to allow for a

very sensitive voltage range to show the ripple.  Calculations

can be corrected to include the 12 volt DC as well as the AC

component.

Running O-Scope

O-Scope I and O-Scope Ip have several operating modes.  On

starting, the program enters the sweep mode where data is

continually captured and displayed.  The two basic modes are

sweep and spectrum.  The F key toggles between the frequency

spectrum and the sweep mode.  O-Scope Ip has several additional

modes not present in the O-Scope I software.  These include a

pause/review mode, main menu mode, log view mode and roll mode.

Sweep and Spectrum Modes

The sweep mode displays input voltage as it changes in time.

This is the classic purpose of the oscilloscope, to show how a

voltages changes in time.  The spectrum mode shows the

frequencies of the input signal that were detected during the

sweep.  The spectrum shows the relative power of the signal at

each frequency.

Pause/Review Mode

On O-Scope I, pressing the <space bar> pauses the sweep so that

you can view the captured wave form.  It will stay paused until

another key is pressed.  For O-Scope Ip, this has been expanded

to become the pause/review mode which will remain in effect until

the <space bar> is pressed again.

In the pause/review mode for O-Scope Ip, most items can be

changed.  Additional sweeps can be run using the S key to start a

single sweep.  This mode must be used to view more than 10

divisions of an extended sweep size.  Changing sweep rates result

in the erasing of the current sweep, but most other items do not

destroy the currently captured sweep.

Main Menu (O-Scope Ip only)

The main menu or setup menu stops the sweep operation and allows

you to change a variety of items.  Included in these are the

screen title and the help line footer which can be used for

making notes about the sweep.  Configuration information can be

saved or loaded.  You can access DOS to copy files or other

simple tasks.  Settings for #divisions, location, delay time, and

voltage offset can be entered rather than slowly changed.  File

names for logging data to disk or viewing previously logged data

can be specified.

This setup menu will remain in effect until option 0, is pressed.

To change value, enter the associated number or letter.  The

current value will appear down the screen and it may be erased

using the <backspace> key.  Changes can be aborted by pressing

the <Esc> key instead of the <Enter> key.  An error message is

displayed if a non existent file name is entered.

Roll Mode (O-Scope Ip only)

The R key starts the roll mode.  For slower sweep rates, usually

100 ms/div. or slower, O-Scope Ip offers a roll mode.  This is

where data is continually captured and rolled to the left of the

window, like a chart recorder.  This mode will fill up to the

number of divisions in the sweep length.  When any key is

pressed, the roll mode ends and the program enters the

pause/review mode.  This allows you to look at the most recent

data.

View Mode (O-Scope Ip only)

View mode provides the ability to load in previously captured log

files.  In view mode you can move through long sweeps or view

logged spectrums.  Most all of the screen information is saved in

the log file.  The + and - keys move from one sweep to the next.

These sweeps are kept one sweep per file and use sequential

numbers for the 3 letter file extension.  An indicator in the

upper left hand corner of the text window indicates the file

extension currently being viewed.  The view mode is entered and

exited using the v key.

Log Option

It is possible in O-Scope I and O-Scope Ip to capture data to

disk.  This data is in human readable form and suitable for

importing into other programs such as spread sheets for

additional processing or custom printing.  The L key toggles the

log option while in the sweep mode.  All traces will be written

to disk while the log option is in effect.  Note that there are

differences between the way O-Scope I and O-Scope Ip log data.

In O-Scope I, all data is logged to one file.  Each sweep begins

with a small header containing the volts per division and sample

rate.  Sweep data consists of a series of voltage numbers with

one sample per line.  In O-Scope Ip, the header has substantially

more information.  Unless the file name is specified with a non

numeric extension, each sweep will be captured to a new file

name.  Note that the view mode can only view one sweep in a file.

O-Scope Ip offers one more capability in logging.  When in the

pause/review mode, pressing the L key after a single sweep or

paused sweep will result in only that sweep being logged.  The

same sweep data may be logged more than one time since it will be

logged each time the L key is pressed.  It is even possible to

switch between the spectrum mode and the sweep mode using the

same captured data if the sweep length is set sufficiently long.

Configuration File

For O-Scope Ip, a configuration file can be created that will set

sweep rates, voltage ranges, trigger mode and other settings to

custom values.  One of O-Scope Ip's command line parameters

allows for specifying a configuration file name.  The setup menu

also allows configurations to be loaded or saved.  Configuration

information is kept in the same format as the log file's header

so it is possible to create a configuration file from an O-Scope

Ip's previously captured log file.

Calculations

The right side of the text window has several pieces of

information.  The upper right contains the time and date for the

end of the captured sweep currently displayed.  Additionally,

various voltages and a period and frequency are calculated for

the wave form.  These provide the DVM and frequency counter

capabilities of the O-Scope.

Voltage calculations include Vdc, Vpeak, Vmin, Vpp, and Vrms.

Vdc is the average or DC voltage observed in the sweep.  Vpeak is

the maximum voltage attained and Vmin is the minimum attained.

Vpp is the peak to peak voltage (Vpeak-Vmin).  Vrms is the AC

voltage or root mean square of the voltage.  These calculations

are made from data in the first 10 divisions of the sweep.

During the calculation program section, the midpoint of the wave

form is calculated and the zero crossings through this point are

determined.  These provide period information which is also used

to calculate frequency.  When there is only a half cycle present

(a single pulse), the frequency is set to 0 and a pulse width is

calculated for the period.  When at least a full cycle is

present, both the period and frequency are calculated.

These calculations can be expected to do a good job when the wave

form on the screen is easy to view.  They can do a reasonable job

even when the wave forms are hard to view.  It is always best to

view the frequency and period calculation critically and not

always just accept these values since the calculations are

subject to noise as well as to sampling constraints.

Familiarization Experiments

I. Getting started, a short guided tour

1. Start the demo selecting the O-Scope I option.  It should come

up AC coupled at 10 V/div and 100 msec/div.  The demo should come

up with values of a sine wave with 5.1Hz frequency, a 6.1V peak

voltage, and there is a 0.5 volt DC offset not visible.

2. Note that there is no Vdc voltage present.  Note that the

bottom of the left text window column reads "AC Sweep X1".  Press

the D key and note the change to "DC Sweep X1".  there should be

a Vdc value of about 0.5 volts.  You may or may not see a small

change in the position of the sine wave.

3. Move the *> cursor to the vertical sensitivity (V/Div.)

setting and press the right arrow key to increase the sensitivity

to 5 V/Div.  Note that the wave form is higher offset with the

peak extending up to about 1.4 divisions above the center line

and descending only about 1.2 divisions below.  Note that the

minor tick marks are each 0.2 divisions.  Press the A key to view

the signal using AC coupling.

4. Press the 7 key twice.  Press the D key to return to DC coupling.

The wave form should now be peaking at the second main division.

Vdc should show 3.75 Volts.  Press the A key to return to AC

coupling, centering the wave form on the 0 line.

5. Move the cursor, *>, down to the sweep rate.  Press the Right

Arrow key and observe the change on the next sweep.  Repeat this

several times, until you reach 500 usec/Div.  Notice how the wave

form spreads out.  Press the Left Arrow key several times, until

the sweep rate reads 100 mV/Div again.

6. Press the X key and note that "AC Sweep X1" has changed to "AC

Sweep X2".  Press the X key a second time to get "AC Sweep X5".

Note that the sample points are spread out to every 5th pixel on

the screen.  Also note that the time per division now reads 20

ms/Div.

7.  Press the <Space Bar> once.  Note that the sweep has stopped.

You should be able to see the separation between the sample

points.  Note that most of the sweep is dots while some dots are

connected by lines.  This is occurs because lines only connect

dots separated by over 3 vertical pixels.  Press the <Space Bar>

to resume sweeps and press the X key to return to X1.  Press the

A key to return to AC mode.

8. Press the Left Arrow key again to lower the sweep rate to 200

mSec/Div.  Note that each wave form is smaller and that the peaks

have no connecting lines while most of the wave form has

connecting lines.  Press the C key to toggle the connecting

lines.  Note that the entire sweep consists only of points.

These are the actual sample points captured.  Press the X key

twice to expand to X5.  You now see only the points separated by

5 pixels.  Press the C key to restore the connecting lines.  Then

press the X key to return to X1 expansion.  In Most cases the

connecting lines will help to understand the wave pattern.  They

can be removed to see only sample points.

9. Press the Left Arrow key and view the wave form.  Repeat this

several times until you reach the 10 sec/div sweep rate.  View

the wave form with and without the connecting lines for these

various sweep rates.  Notice how the wave form becomes

unrecognizable until you reach 10 sec/div and then it appears as

a sine wave again.  This is called aliasing.  If the sweep rate

is too low for the signal, it is possible to see something that

is not really present.  In this case it appears as a 0.1 Hz wave

form instead of the 5.1 Hz wave that really is there.

10. Press the Right Arrow key to return to 5 Sec/Div.  Notice how

the wave form didn't just double.  Instead it shows a strange

pattern.  This is how you can usually tell if a signal is

aliased.  By increasing the sweep speed, a normal non aliased

wave form will spread out.  An aliased wave form will eventually

show a strange pattern.  Press the Right Arrow key several times

to return to 100 ms/Div sweep rate.

11. Move the cursor, *>, down to "Trigger Off" by pressing the

Down Arrow key twice.  Press the Right Arrow Key observe the

trigger mode.  It should read "Trigger Slope +".  This is for a

rising signal level.  Note that wave form is starting from the

zero line and going positive.  Press the Right Arrow key again.

The Trigger mode should indicate "trigger Slope -".  Note now

that the wave form is beginning at 0 and is first going negative.

Press the Left Arrow key to select "Trigger Slope +" again.

12.  Move the cursor, *>, up by pressing the Up Arrow key once.

and press the Right Arrow key several times.  Note the small "-"

tick mark rising a little each time you press the Right Arrow

key.  Adjust this tick mark until it is half way between the zero

line and the first division line above zero.  Use the Right and

Left Arrow keys to make the adjustment.  Notice that the trigger

point is still beginning at the tick mark and going positive.

Note that sometimes when pressing a key that the trigger point is

arbitrary.  This is because keystrokes will cause a trigger no

matter what the wave form is doing.

13.  Press the Right arrow some more to raise the trigger point

to half way between the first and second divisions.  You should

still be at 5 V/Div with the wave form peak at 6.1 Volts and in

the AC coupling mode.  The DC offset should still be 3.75 Volts

This point should be above the top of the wave form.  Note that

each time you press a key, there will be a trace which will erase

shortly after the sweep is complete.  The O-Scope is not sweeping

because it is waiting for a trigger which is beyond the incoming

signals wave form.  Press the D key to select DC coupling.

Notice that the sweep is occurring since the wave form now

extends up to the second division above the zero line.

14. Move the cursor, *>, down to Hold Time using the Down Arrow

key.  Use the Right Arrow key to increase the Hold time to 1000

milliseconds (1 second delay).  Observe that the sweep cycle is

delaying after each sweep.  Use the <Pg Dn> key to speed up the

process of increasing the time.  After looking at the hold off

delay time, use the <Pg Up> key to quickly reduce the value to 0.

Note that there is always a short delay.

15. Move the cursor, *>, down to the vertical position, Vert.

Pos., spot.  Use the Left Arrow (or <Pg Up>) key to move the

trace down.  Note that the voltage calculations do not change.

Also, note the trigger tick mark move down with the wave form.

The trigger point occurs at the same point on the wave form.

16. Change the sweep rate to 20 ms/div and press the 5 key 3

times to increase the waveform frequency to 240 Hz.  Press the F

key and observe the frequency spectrum.  For a moderately slow

machine without math coprocessor this could take many seconds.

Note that at the bottom of the graphics window, the numbers range

from D.C. to 1250 Hz.  Notice that the main peak is just to the

left of the first tick mark which is 250 Hz.  Notice that this

peak reaches 0 and that the base line for the frequency is about

half way between the 45 dB and 67 dB tick marks on the power

axis.  This corresponds to about 55 dB.  With a pure sine wave, a

base line with no peaks rising above the 55 dB mark indicates a

THD of about 1/4 percent.  A peak rising to 40 dB would indicate

a distortion of about 1%.  Press the F key to return to the sweep

screen.

17. Press the 9 key.  The wave form should switch to a square

wave.  Turn off the trigger and move the vertical position to 0.

Set the coupling to AC.  The square wave should go from the zero

line up to about 1.2 divisions.  Press the F key to view the

spectrum which should contain many harmonics.  Press the F key to

return to the sweep window.

This concludes the basic introduction.

Appendix A - Printing out Graphics Screens

It is possible to print out DEMO1P.EXE screens at any time.

Before running DEMO1P.EXE, you must load GRAPHICS.COM which comes

with DOS.  See your DOS manual for setup information for

printers.  Note that very old versions of DOS may not have a

GRAPHICS.COM which is able to print VGA or Monochrome graphics

modes or be compatible with your printer in graphics mode.

================================================================

Example - Using default values for GRAPHICS.COM

C:\>graphics.com<Enter>

C:\>cd oscp<Enter>

C:\OSCP>demo1p<Enter>

================================================================

If GRAPHICS.COM is run (loaded) before running DEMO1P.EXE, it is

possible to print the O-Scope screen at any time by pressing the

<Prt Sc> key.  The program activity will stop while printing.

NOTE - unless you enter the pause mode (by pressing the <Space

Bar> ) it is very likely that some part of the screen will be in

the process of updating and that some data will not be on the

screen during the printing and will be missing from the printout.

The printout will occur using the data that is on the screen at

exactly the time you press the key.  The other alternative to

entering the pause mode is to increase the Hold time to over 1/2

second so that the program is idle long enough to press the key.

###############################################