O-Scope I API

Version 1.0

Introduction

O-Scope I software is now divided between the hardware controlling driver

program, OSCP1TSR.EXE, and the control and display program, OSCOPE1P.EXE.

A significant benefit to this for the more sophisticated O-Scope I users is

that one can now create customized data acquisition programs for special

uses.  Rather than having to write a sophisticated program which is very

hardware dependent and time sensistive to collect the data, one can now

use the O-Scope I hardware driver program to handle this portion of the

job.

Using OSCP1TSR.EXE, one can control the O-Scope I hardware from most any

PC based programming language by using the API, which stands for

Applications Programming Interface.  A simple set of interface commands

allows for setting up hardware and for capturing data.

Overview

O-Scope I version 2 operation is achieved via two programs, OSCP1TSR.EXE and

OSCOPE1P.EXE.  OSCP1TSR.EXE, the TSR, is loaded first and provides control

over the hardware.  OSCOPE1P.EXE is run after the TSR and provides the

human interface, the displays and commands.  It also provides control over

the TSR to do the data collection.  Note that this is the portion of the

software that makes O-Scope I a digital storage oscilloscope instead of

a data logger.  By writing a different program, one could create a data

logger instead of an oscilloscope.

The API is the definition of the interface between these two program

sections which make up O-Scope I.  By using the API interface commands,

the control program sets up and configures the TSR.  It then commands the

TSR to capture and transfer the captured data.

On startup, the TSR captures an interrupt vector, default is 120 (78Hex),

which is used by the control program to communicate with the TSR.  A data

buffer is also reserved for local data capture.  Both the interrupt vector

number and the size of the data buffer may be specified on the command line

of the TSR (along with the printer port's data address).

API

The API commands are accomplished by using the interrupt, INT86, functions,

which are available in most PC programming languages.  These functions load

the processor's registers with calling parameters, call a specified

interrupt vector and then return saving the register's contents in a set

of return parameters.  The calling and return parameter data details are

provided in the programming language manuals.  The interrupt vector number

default is 120 (78H hexadecimal), but it can be set between 120 and 127

(78H - 7fH) on the command line of the TSR driver.

The INT86 interrupt functions directly reference the names of the registers

for the 80x86 processor.  Names such as AX, BX, CX, DX, SI, and DI are

used.  These utilize 16 bit integer values.  For some purposes, these

values are split into two 8 bit byte values, a high byte and a low byte.

Typically register AX is called ax for the 16 bit word or ah and al for

the high byte and low byte.

Accessing the TSR from a user program consists of several parts.  First, the

calling information is loaded into the calling register data array.  second,

the appropriate interrupt vector is called via the INT86 function.  Third,

the desired data is removed from the return register data array.

Different API functions (or subroutines) are used for the different

activities needed.  These different functions are determined by the value

of the byte register ah, the top half of register AX.  A list of functions

is provided below by number.  Note that this number is given in hexadecimal

which is 0-9 then a-f before function 10H occurs.  A brief description

for each implemented function is also provided on the same line.

API Functions List

func0    - return status

func1    - reset

func2    - set up parms

func3    - set vert. sens.

func4    - set AC/DC

func5    - set sweep rate #

func6    - set trig mode/level

func7    - set # samples to capture in a sweep

func8    - generate a sweep, wait for fast, return for slow

func9    - start a triggered sweep  "

funca    -not currently implemented

funcb    -not currently implemented

funcc    -not currently implemented

funcd    - activate interrupts

funce    - deactivate interrupts

funcf    - shut down tsr

func10   -not currently implemented

func11   -not currently implemented

func12   -not currently implemented

func13   -not currently implemented

Each function requires certain information to be placed into the calling

register array.  Values are then returned in the return register array.

Typically, the function select is in AH with AL perhaps containing some

additional information and on return an error code is in the return

register array's AX parameter.  AL is often used as a subfunction parameter.

ah contains the function #  - NOTE that ah is not shown specifically below

ax will return the error code 0-ok

func0    -return status

subfunc al = 0

bx is status

cx is # new samples available

dx is maximum sample buffer size

conditions

1. # of new samples available

subfunction al = 1

bh # of sweep rates

bl # for fastest slow sweep rate

subfunction al = 2

same as subfunc 1 with measured sweep rates table pointer

pointer to measured sweep rates table   //oscope 1

a far pointer to a table in the application is in registers ES:DI

the table values are 32 bit unsigned values and contain one more entry

than the number of time bases returned in subfunction 1 register BH.

func1 -reset (not implemented)

clear Sample buffer, shut off capture data in intr.

func2 -set up parms (not implemented yet)

set vert sens, ac/dc, sweep speed, samples to capture

bh vert sens + ac/dc, bl sweep speed

func3 -set vert. sens.

al bits 0-6 channel #

bl = 0 - N sensitivity ranges (0 least sensitive 7 most sensitive)

func4 -set AC/DC

al bit 7 = 0 AC, 80h DC

al bits 0-6 channel #

func5 -set sweep rate #

al has sweep mode

bx has time value

sweep mode = 0

select standard sweep range # bx

sweep mode = 1

select custom high speed time value where

bx is the # of delay loop counts between acquisitions (actual time depends

on the computer)

sweep mode = 2

select custom low speed time value where

bx is the # of milliseconds between data acquisitions

func6 -set trig mode/level

al = mode

0 none

1 + int

2 - int

3 + ext

4 - ext

bx is level  (signed)

func7 -set # samples to capture in a sweep

bx = # of samples (after trigger) to end of sweep

0 is continual (for slow sweeps only)

func8 -generate a trigger, wait for fast, return for slow (not implemented)

command a sweep (don't wait for trigger) (for slow sweeps)

func9 (union IREGS far  * regsptr); //start a triggered sweep  "

al

0 standard sweep no trigger

1 sweep to wait for trigger

2 pretrigger data capture  (not implemented)

bx has delay (# of samples from trigger)

start a triggered sweep (fast must delay for whole sweep, slow can return)

funca -end sweep (slow sweep only for oscope1) (not implemented)

funcb (union IREGS far  * regsptr); // read n samples from  buff

al selects from where

0 next n samples from last read (cleared by generate a sweep

1 from beginning of trigger event    (not implemented)

2 before trigger event               (not implemented)

3 1/2 before trigger, 1/2 after trigger  (not implemented)

bx # of samples

cx is sample  # for trigger point

es:di destination

funcc check status of sweep

returns tracedone flag in ax, 0 done, 1 in process, 2 waiting for trigger

funcd -set or activate the timer interrupt (a required set up)

funce -clear or deactivate the timer interrupt (a shut down function)

funcf -shut down tsr (not implemented)

terminate tsr - remove from memory

func10 -start delay counter value (0-32767 milliseconds)

func11 -test delay counter

bx is value 0-32767 milliseconds

returns ax with difference -1 to -32768 time hasn't happened

0 - 32767 time has expired between just now and 32 seconds ago

func12 (not implemented)

func13 (not implemented)